Introduced by Council Members Jones, Joost, Johnson, Redman, Brown, Holt, Shad, Davis, Fussell & Gaffney, substituted by the Land Use and Zoning Committee, and amended twice on the floor by Council:

ORDINANCE 2009-401-E

AN ORDINANCE AMENDING SECTION 326.103 (EXEMPTIONS), ORDINANCE CODE, & SECTION 656.1303 (ZONING LIMITATIONS ON SIGNS), ORDINANCE CODE, TO PERMIT ADVERTISING ON TRANSIT SHELTERS ON PUBLIC RIGHTS-OF-WAY, PUBLICLY OWNED PROPERTY, OR PRIVATELY OWNED PROPERTY; AMENDING SECTION 326.104 (UNLAWFUL SIGN STRUCTURES), ORDINANCE CODE, TO CROSS REFERENCE SECTION 326.103 (EXEMPTIONS), ORDINANCE CODE, AS AMENDED;  AMENDING SECTION 656.1333 (SIGNS PERMITTED), SECTION 656.1334 (SIGNS EXEMPTED) & SECTION 656.361.20 (STREETSCAPE DESIGN STANDARDS), ORDINANCE CODE, TO LIST CRITERIA APPLICABLE DOWNTOWN;  AMENDING SECTION 307.106 (APPROVAL OF CHANGES TO LANDMARKS, LANDMARK SITES, AND PROPERTY IN HISTORIC DISTRICTS; APPLICATION PROCEDURES), ORDINANCE CODE, REGARDING HPC APPROVAL OF TRANSIT SHELTER DESIGN TYPES; PROVIDING FOR SEVERABILITY THROUGHOUT; ATTACHING CURRENT TEXT OF ORDINANCE CODE SECTIONS FOR REFERENCE; PROVIDING AN EFFECTIVE DATE.

WHEREAS, Section 337.408, Florida Statutes, permits transit shelters, including advertising displayed thereon, within the right-of-way limits of any municipal, county, or state road, provided that such transit shelters are for the comfort or convenience of the general public or are at designated stops on official bus routes, as more particularly described in Revised Exhibit 1, attached hereto; and

WHEREAS, on January 6, 2009, in Metro Lights, LLC v. City of Los Angeles, the United States Court of Appeals, Ninth Circuit, held that permitting advertising on transit shelters on public rights-of-way or on transit shelters on public property, while otherwise banning offsite signs to promote traffic safety and aesthetics as substantial government interests, is not unconstitutionally underinclusive under the First Amendment of the United States Constitution, as more particularly described in Exhibit 2, attached hereto; and

WHEREAS, while the Council of the City of Jacksonville remains dedicated to protecting the substantial governmental interests of promoting traffic safety and aesthetics which underlies the Charter and Ordinance Code provisions regarding offsite commercial signs, the Council is also mindful of the substantial governmental interests in providing safe, efficient and convenient transit service to the community, and maximizing the use of public dollars for such purpose; now therefore,

BE IT ORDAINED by the Council of the City of Jacksonville:

Section 1.		Section 326.103 (Exemptions), Ordinance Code, & Section 326.104 (Unlawful Sign Structures), Ordinance Code, Amended.  Section 326.103 (Exemptions), Ordinance Code, & Section 326.104 (Unlawful Sign Structures), Ordinance Code are hereby amended, in part, to read as follows:

CHAPTER 326. SIGNS AND OUTDOOR DISPLAY STRUCTURES.

PART 1. GENERAL REGULATIONS.

*  *  *

Sec. 326.103.	Exemptions.

*  *  *

(p)	Signs on transit shelters located on public rights-of way, publicly owned property, or privately owned property, all subject to Section 326.108 (Zoning Limitations on Signs).

In the event that any portion of this section, including any exception contained herein, is declared invalid, unenforceable, unconstitutional or void, or is permanently enjoined, or if the existence of any provision of this section would result in any other portion of this Chapter or Chapter 656 or Article 23 of the Charter being held to be invalid, unenforceable, unconstitutional or void, and the court does not sever such invalid portion of this section, then the invalid portion of this section is repealed and invalid and thereafter no signs of the type included within the exemption shall be erected without compliance with the remainder of this Chapter and this Ordinance Code. It is the specific intent that the invalidity of any portion of this section shall not affect any other section, subsection, paragraph, subparagraph, sentence, phrase, clause or word of this Chapter, Chapter 656, Article 23 of the Charter, or this Ordinance Code.

# *  *  *

Sec. 326.104.	Unlawful sign structures. It shall be unlawful and a violation of this Building Code to erect, alter or maintain a sign which:

* * *

(d)	Is placed in a public space or street right-of-way, except as provided in Section 326.103(p), and that subdivision identification signs may be installed under the provisions of Sections 746.107 and 656.1308.

* * *

# Section 2.		Section 656.361.20 (Streetscape design standards), Ordinance Code, Amended.  Section 656.361.20 (Streetscape design standards), Ordinance Code, is hereby amended, in part, to read as follows:

# CHAPTER 656. ZONING CODE.

*  *  *

# PART 3. SCHEDULE OF DISTRICT REGULATIONS.

*  *  *

SUBPART H. DOWNTOWN OVERLAY ZONE AND DOWNTOWN DISTRICT REGULATIONS.

*  *  *

Section 656.361.20.	Streetscape design standards.

Purpose and Intent. The streetscape design standards are established to provide design criteria which require a certain level of quality; enhance street level design to attract pedestrian use; develop a system of pedestrian-oriented streets and walkways; improve pedestrian and transit links among key activity centers and districts; emphasize, protect and enhance entrances and edges of the central business district subdistricts; promote continuity between public and private developments; provide for protection of air quality through the mitigating effects of trees and provide shade and enhance the appearance of the central business district. All new buildings and structures, and rehabilitation of existing buildings and structures, and any other proposed projects, public or private, that would affect, modify or change the streetscape shall meet the following criteria:

(a)   A streetscape shall be constructed in accordance with the provisions of this subsection and the design standards set forth in the Downtown Jacksonville Streetscape Standards, including Downtown Sidewalk Utility Design Standards, a copy of which is on file with the Legislative Services Division, the City Engineer's Office and the JEDC, which are hereby adopted as the streetscape design standards for the Downtown Overlay Zone, whenever any new building or structure is erected in the central business district or whenever any existing building or structure undergoes major renovation, or whenever any improvements are made to the streetscape. The streetscape shall include the following items, unless the proposed project is only for improvements to the streetscape, then only the proposed changes to the streetscape shall adhere to the following, except as provided in clause (xi) of subsection (5), or unless staff determines that the proposed changes to the streetscape are substantial enough to warrant the entire streetscape affected to be improved to the full standard:

*  *  *

(5)   Transit Shelters. Transit shelters shall be installed where appropriate. The type and design, number, spacing and location of transit shelters shall be approved by the DDRB pursuant to Section 656.361.9(d)(2)(iii) and comply with the standards set forth in the Downtown Jacksonville Streetscape Standards, and as approved by the City’s Engineer, if required.  The transit shelter design type shall be chosen from at least three options provided by JTA, whose design shall be in keeping with the historic character of the downtown streetscape furniture and street lights.  Such approval by the City’s Engineer, if required, shall occur before final approval of the transit shelter by the DDRB. Any changes to the transit shelter after the DDRB approval by JTA and/or the City’s Engineer shall be communicated to the DDRB, and approval granted by the DDRB prior to the City’s Engineer approving such changes.  The following criteria shall apply:

(i) 	The permitted transit shelter site, excluding overhangs, shall be no larger than 16 feet long by 10 feet high by 5 feet wide, and no larger than 80 square feet in size.

(ii) The overhang shall have a minimum clearance of 8 feet and be set back from the back of curb a minimum of 2 feet.

(iii)  	 Site location of transit shelters must provide a minimum of 6 feet clearance to pedestrians on the sidewalk or other surface allowing for pedestrians.

(iv)  	 Transit shelters must be at least 6 feet from any driveway.

(v)  	 	Transit shelters shall only number one per side of street per block, except the bus rapid transit stations as shown on JTA’s Jacksonville BRT Phase I map, a copy which is on file with the JEDC.

(vi)   	 Transit shelters must be a minimum of 10 feet from an intersection or crosswalk.

(vii)  	 Transit shelters must be a minimum of 6 feet from any building entrance or exit.

(viii)   	 Transit shelters may not obstruct any view of traffic or roadway signage.

(ix)	 	Transit shelters may be illuminated with lighting that is interior to the structure and shall not interfere with the ability of vehicular users of the road to read traffic signs or see traffic signals.

(x)		Transit shelters shall include, at a minimum, one trash can per shelter and the trash can shall not obstruct the minimum 6 feet requirement for pedestrian clearance on the sidewalk or other surface allowing for pedestrians.

(xi)		Transit shelters that disrupt more than 50% of any one side of the street, per block, shall meet the Downtown Streetscape Design Standards for the side of the street, per block, that is disrupted.

(xii) 	Existing transit shelters shall not be required to meet these standards until replaced with a new transit shelter, which shall meet the requirements of clauses (i) through (xii) and (xiii) of this subsection.

(xiii)	Any signage associated with transit shelters and associated areas shall be subject to Chapter 656, Part 13, Subpart B.

(xiv)		In the event that any portion of this section, including any exception contained herein, is declared invalid, unenforceable, unconstitutional or void, or is permanently enjoined, or if the existence of any provision of this section would result in any other portion of this Chapter or Chapter 326 or Article 23 of the Charter being held to be invalid, unenforceable, unconstitutional or void, and the court does not sever such invalid portion of this section, then the invalid portion of this section is repealed and invalid and thereafter no signs of the type included within the exemption shall be erected without compliance with the remainder of this Chapter and this Ordinance Code. It is the specific intent that the invalidity of any portion of this section shall not affect any other section, subsection, paragraph, subparagraph, sentence, phrase, clause or word of this Chapter, Chapter 326, Article 23 of the Charter, or this Ordinance Code.

# Section 3.		Section 656.1303 (Zoning Limitations on Signs), Ordinance Code, Section 656.1333 (Signs Permitted), Ordinance Code, & Section 656.1334 (Signs Exempted), Ordinance Code, Amended.  Section 656.1303 (Zoning Limitations On Signs), Ordinance Code, Section 656.1333 (Signs Permitted), Ordinance Code, & Section 656.1334 (Signs Exempted), Ordinance Code, are hereby amended, in part, to read as follows:

# CHAPTER 656. ZONING CODE.

*  *  *

# PART 13. SIGN REGULATIONS.

*  *  *

Sec. 656.1303.	Zoning limitations on signs.

*  *  *

(k)	Special exemptions:

*  *  *

(3)	Signs on transit shelters located on public rights-of way, publicly owned property, or privately owned property, subject to the following criteria and superseding any other provisions in Chapter 656, except as provided in Section 656.361.20.

(i)		Transit shelters with advertising and internal sign illumination are not permitted on or adjacent to AGR, CSV, ROS, or RR-Acre zoning districts, and are not permitted within PUD zoning districts in AGR, CSV, ROS, or RR land use categories.

(ii)		Transit shelters with advertising and internal sign illumination are permitted by sign waiver on or adjacent to all RLD zoning districts, RMD-A and RMD-B zoning districts, PUD zoning districts in an LDR land use category, and adjacent to schools and parks in a PBF land use category.

(iii)		Transit shelters with advertising and internal sign illumination are permitted by right on or adjacent to RMD-C, RMD-D, CO, and CRO zoning districts if the transit shelter location is on an identified corridor according to the JTA Corridor Route Map, and the transit shelter is at least 200 feet from a single family use.  All others on or adjacent to RMD-C, RMD-D, CO, and CRO zoning districts are permitted by sign waiver.

(iv)		Transit shelters with advertising and internal sign illumination are permitted by right on or adjacent to RMD-MH, RHD-A, RHD-B, CN, CCG-1, CCG-2, IBP, IL, IH, IW, PBF-1 (except schools and parks), PBF-2 (except schools and parks), PUD (unless otherwise limited herein), and PUD-SC (unless otherwise limited herein) zoning districts, and within the CBD land use category.

(v)		Transit shelters with advertising and/or internal sign illumination are not permitted in the San Marco Overlay Zone, Riverside Avondale Zoning Overlay, the Riverside Avondale Historic District, the Springfield Zoning Overlay, the Springfield Historic District, or any other locally or nationally designated historic districts.

(vi)		Signs shall not exceed 24 square feet in size per transit shelter with a dimension no higher or wider than the size dimensions of the shelter side that the sign is to be placed or attached to, below the bottom of the roofline and the sidewalk, or other surface, the shelter is attached to.

(vii)		Signs shall not exceed a maximum of two (2) signs on a transit shelter structure.

(viii)	Transit shelters with advertising and internal illumination shall not be deemed non-conforming uses if its zoning district or the adjacent zoning district changes after installation of the transit shelter.

(ix)		JTA shall report each year to the City Council to provide an assessment of transit shelters with advertising. The report shall document the transit shelter construction locations, and provide a correlation based on ridership demand and special needs.

(x)		In the event that any portion of this section, including any exception contained herein, is declared invalid, unenforceable, unconstitutional or void, or is permanently enjoined, or if the existence of any provision of this section would result in any other portion of this Chapter or Chapter 326 or Article 23 of the Charter being held to be invalid, unenforceable, unconstitutional or void, and the court does not sever such invalid portion of this section, then the invalid portion of this section is repealed and invalid and thereafter no signs of the type included within the exemption shall be erected without compliance with the remainder of this Chapter and this Ordinance Code. It is the specific intent that the invalidity of any portion of this section shall not affect any other section, subsection, paragraph, subparagraph, sentence, phrase, clause or word of this Chapter, Chapter 326, Article 23 of the Charter, or this Ordinance Code.

*  *  *

SUBPART B.  DOWNTOWN SIGN OVERLAY

*  *  *

Sec. 656.1333.	Signs permitted.

*  *  *

(f)   Pylon/pole, roof, and monument signs, and transit shelter signs:  Allowed only by special exception approved by the Downtown Development Review Board using the criteria set forth in subsection (2) of this Section, except transit shelter signs. Unless otherwise provided in the special exception, all pylon/pole signs shall meet the setback requirements of Section 656.1303(i). Transit shelter signs shall meet the requirements in Section 656.1333(h) and shall not be eligible for a special sign exception under Section 656.1331(j), but shall only be allowed by special exception following the criterion outlined under Section 656.1333(h).

*  *  *

(h) Transit shelter signs with advertising: Transit shelter signs with advertising shall be subject to Section 656.1333 (f) and shall only be permitted subject to the following criteria:

(1) 	Signs do not exceed 24 square feet in size per transit shelter with a dimension no higher or wider than the size dimensions of the shelter side that the sign is to be placed or attached to, below the bottom of the roofline and the sidewalk, or other surface, the shelter is attached to.

(2) 	Signs do not exceed a maximum of two (2) signs on a transit shelter structure

(3) 	Advertisement content is consistent with Jacksonville Transportation Authority’s advertising policy dated October 26, 2006, a copy of which is on file with the JEDC. Any modifications to this policy that affects downtown shall be reviewed and approved by the DDRB.

(4)	Transit route information signs shall not exceed 6 square feet in size and shall meet the same clear view zone dimensional requirements as universal parking signs pursuant to Section 656.1333(d)(2)(C), and shall not obstruct the minimum 6 feet requirement for pedestrian clearance on the sidewalk or other surface allowing for pedestrians.

(5) DDRB staff shall be authorized to review changes to advertisement  content and design within a previously DDRB approved transit shelter and associated advertisement  signage plan provided the location and size of the advertisement signage plan has not changed and the advertisement signage plan is consistent with the Jacksonville Transportations Authority’s advertisement policy as approved by DDRB. If DDRB staff determine a proposed advertisement to be changed out is not consistent with the Jacksonville Transportation Authority’s advertising policy, then the Jacksonville Transportation Authority shall not place the proposed advertisement on the affected transit shelter until said advertisement is considered by DDRB staff to be consistent with said advertisement policy.

(6)	In the event that any portion of this section, including any exception contained herein, is declared invalid, unenforceable, unconstitutional or void, or is permanently enjoined, or if the existence of any provision of this section would result in any other portion of this Chapter or Chapter 326 or Article 23 of the Charter being held to be invalid, unenforceable, unconstitutional or void, and the court does not sever such invalid portion of this section, then the invalid portion of this section is repealed and invalid and thereafter no signs of the type included within the exemption shall be erected without compliance with the remainder of this Chapter and this Ordinance Code. It is the specific intent that the invalidity of any portion of this section shall not affect any other section, subsection, paragraph, subparagraph, sentence, phrase, clause or word of this Chapter, Chapter 326, Article 23 of the Charter, or this Ordinance Code.

(ih)   Sign area computation:  For signs erected pursuant to this Section, the area of each sign surface shall be calculated as provided in Section 656.1302(u).

(ji)   Special sign exceptions:  The Downtown Development Review Board may approve special sign exceptions to Section 656.1333 provided the proposed sign plan shows, in addition to the criteria set forth in Section 656.1335 and Section 656.1303(c):

*  *  *

(kj)   Other signs prohibited:  Any sign not specifically allowed in this Section or exempted under Section 656.1334 shall be prohibited.

(lk)   Compliance with Building Codes:  In addition to meeting the requirements of this Subpart, signs in the Downtown Sign Overlay Zone shall also meet all applicable requirements of the Florida Building Code and the City of Jacksonville Building Code - Administrative Provisions.

Sec. 656.1334.	Signs exempted.

(a)	The following signs do not require permits or fee payment but are subject to design review by the Downtown Development Review Board Development Authority:

*  *  *

(4)		Signs on transit shelters located on public

rights-of-way, on publicly owned property, or privately owned property, as permitted by Section 656.1303 and subject to Sections 656.361.20 and 656.1333.

(b)	The following signs do not require permits, fee payment, or design review by the Downtown Development Review Board Development Authority, so long as such signs meet the requirements of the Sections indicated:

*  *  *

(c)	The following signs do not require permits, fee payment, or design review by the Downtown Development Review Board Development Authority:

*  *  *

Section 4.		Section 307.106 (Approval of changes to landmarks, landmark sites, and property in historic districts; application procedures), Ordinance Code, amended.    Section 307.106 (Approval of changes to landmarks, landmark sites, and property in historic districts; application procedures), Ordinance Code, is hereby amended, in part, to read as follows:

CHAPTER 307. HISTORIC PRESERVATION AND PROTECTION.

PART 1. GENERAL PROVISIONS.

*  *  *

Section 307.106. 	Approval of changes to landmarks, landmark sites, and property in historic districts; application procedures.

*  *  *

(v) The Historic Preservation Commission shall review and approve the transit shelter design type for any locally or nationally designated historic district.  The transit shelter design type shall be chosen from at least three options provided by JTA, whose design shall be in keeping with the applicable Historic District Regulations.  No COA shall be required for any transit shelter which conforms to the transit shelter design type approved by the Historic Preservation Commission.  Pursuant to Section 656.1303(k)(3)(v), transit shelters with advertising and/or internal sign illumination are not permitted in the San Marco Overlay Zone, Riverside Avondale Zoning Overlay, the Riverside Avondale Historic District, the Springfield Zoning Overlay, the Springfield Historic District, or any other locally or nationally designated historic districts.

*  *  *

Section 5.		Conformance with Code.  The Jacksonville Municipal Ordinance Code is hereby amended throughout all sections of the Ordinance Code to conform with the intent of the provisions in Sections 1 through 4 of this ordinance.  The Office of General Counsel along with the Municipal Code codifiers are authorized and directed to ensure code changes are properly codified as approved herein.

Section 6.		Severability Clause.  It is the specific intent of the Council that in the event that any portion of this ordinance, including any exception contained herein, is declared invalid, unenforceable, unconstitutional or void, or is permanently enjoined, or if the existence of any provision of this ordinance would result in any other portion of Chapter 326, Chapter 656 or Article 23 of the Charter being held to be invalid, unenforceable, unconstitutional or void, and the court does not sever such invalid portion of this section, then the invalid portion of this ordinance is repealed and invalid and thereafter no signs of the type included within the exemption shall be erected without compliance with the remainder of the applicable Chapter or Article. It is the specific intent that the invalidity of any portion of this ordinance shall not affect any other section, subsection, paragraph, subparagraph, sentence, phrase, clause or word of Chapter 326, Chapter 656, Article 23 of the Charter, or the Jacksonville Municipal Ordinance Code.

Section 7.		Current Code Attached.  The current text of the Ordinance Code Sections amended by this Ordinance is attached hereto for reference as Revised Exhibit 3.

Section 8. 	Effective Date.  This Ordinance shall become effective upon the satisfaction of all of the following: (i) upon the written agreement (“Defense Agreement”) between the City and Jacksonville Transportation Authority for the defense of the City’s offsite commercial billboard ban in a suit which may be filed challenging the ban because of the exemption for transit shelter advertisements created by this Ordinance; and (ii) upon signature by the Mayor or upon becoming effective without the Mayor’s signature.    The Mayor and Corporation Secretary are authorized to execute the Defense Agreement on the following terms: defense shall be at all levels, including appeal, and cover all costs of litigation and appeal with respect to the issue set forth above; JTA will participate with the City in the costs of defense and will provide up to the first $250,000 of such costs and participate 50/50 thereafter at the option of the City; JTA may choose to utilize the Office of General Counsel, at the current rates and charges available to City departments; in the event that JTA determines at any time to discontinue defense efforts, JTA will tender defense of the action to the City, at which time the City may continue such defense or Council may effect a repeal of this Ordinance in order to moot the action, and JTA shall support either action of the City; and JTA shall incorporate an anti-grandfathering provision in any contract with a shelter advertiser.

Form Approved:

/s/   Shannon K. Eller____________

Office of General Counsel

Legislation Prepared By:	Shannon K. Eller

/tmp/in/input.doc