Introduced by the Council President at the request of the Mayor:

ORDINANCE 2015-731

AN ORDINANCE AMENDING CHAPTER 420 (GENERAL PROVISIONS), PART 2 (OPERATIONAL PROVISIONS), SECTION 420.214 (INSTALLATION, MAINTENANCE, LOCATION, PERMIT AND FEE OF FIRE HYDRANTS), ORDINANCE CODE, TO PROVIDE CONSIDERATION FOR NEW INFRASTRUCTURE’S HYDRANT INSTALLATIONS TO EXCEED DISTANCE REQUIREMENTS SET FORTH IN SECTION 420.214(b), ORDINANCE CODE, WHEN JUDGED TO BE OF A DISPROPORTIONATE EFFORT OR EXPENSE TO THE INCREASE IN FIRE SAFETY; PROVIDING AN EFFECTIVE DATE.

BE IT ORDAINED by the Council of the City of Jacksonville:

Section 1.		Amending Section 420.214.  Section 420.214 (Installation, maintenance, location, permit and fee of fire hydrants), Part 2 (Operational Provisions), Chapter 420 (General Provisions), Ordinance Code, is hereby amended to provide consideration for new infrastructure’s hydrant installations exceeding distance requirements in circumstances in which there is judged to be a disproportionate effort or expense with little increase in fire safety, and as amended shall read as follows:

CHAPTER 420.  GENERAL PROVISIONS.

*  *  *

PART 2. OPERATIONAL PROVISIONS.

*  *  *

Section 420.214.  	Installation, maintenance, location, 						permit and fee of fire hydrants.

(a)	Water supply.  Approved fire hydrants shall be provided for buildings to meet the necessary fire flow requirements as determined by the Director of Fire and Rescue or his or her designee.  Where public water supply is inadequate or not available, an approved alternative water source meeting the fire flow requirements shall be provided. Fire flow performance tests shall be witnessed by the Director of Fire and Rescue or his or her designee.

(b)	Location.  The location and number of hydrants shall be designated by the Director of Fire and Rescue or his or her designee, but in no case shall distance between installed fire hydrants exceed 1,000 feet (305 meters) for new infra-structure. Maximum distance from the nearest hydrant to the most remote exterior point of any building shall be 500 feet (152 meters). The distance shall be measured on a roadway surface meeting the fire department access requirements of Section 321.104, Ordinance Code.

(c)	Exception.  One- and two-family dwellings, including attached or detached accessory structures are exempt from this Section.Where required maximum distance of existing fire hydrants is exceeded for the construction of new structures, consideration may be given when there is judged to be a disproportionate effort or expense with little increase in fire safety. Factors such as fire sprinkler protection, building size, construction type, occupancy type, and setbacks will be used as factors in this determination, but in no case will fire flow be less than the requirements of Section 321.104, Ordinance Code.

(d)	Exception.  One- and two-family dwellings, including attached or detached accessory structures are exempt from this Section.

(d)(e)	A permit shall be required for new or replacement fire hydrants set forth in this section. The fee for such permit is set forth in Section 123.102, Ordinance Code.

Section 2.  	Effective.	  This ordinance shall become effective upon signature by the Mayor or upon becoming effective without the Mayor’s signature.

Form Approved:

/s/   James R. McCain, Jr.

Office of General Counsel

Legislation Prepared By: James R. McCain, Jr.

/tmp/in/input.doc